1/ Install the USB driver from MTK USB Driver_v1.0821.zip, for Win 2K / XP system, please run InstallDriver.exe; For Vista, please run installdrv64.exe.

Click Exit and finish the installation of driver.

2/ Updating, please use FlashTool_v3.1004.00 or a higher version. Here we use the former one.

a) Please extract the file of FlashTool_v3.1004.00.

b) Please run Flash_tool.exe,

c) If there is a security warning like this, please just click the first button

Click OK

c) Connect the special Updating cable to the computer, do not connect to the phone.

d) After step a/b, you will see this screen

Click Download Agent and choose MTK_AllInOne_DA.bin from the folder of FlashTool_v3.1004.0.

e) Click Scatter File and choose scatTELSDA53_09A_GEMINI.txt from the folder of Software. After loading the file, please tick off BOOTLOADER.

f) Click Options and choose USB Download/Readback.

g) Click download

h) Put on the battery to the phone, keep pressing the right key and connect a normal USB cable to the computer and the phone.

i) After 2 minutes, it will show OK which means the updating is successfully finished.