SMC3216IT firmware Flash Process    (PRELIMINARY 6/14/16)

Flashing IT mode in DOS:

1) Please copy all files from DOS folder to bootable USB disk

2) Run SMC3216T.bat and following the instruction to enter the last 9 digits/character of the SAS address.

3) If you are having problem to enter the SAS address, please use the command below to set the controller SAS address.

sas3flsh.exe -o –sasadd “Controller SAS address”

For example: sas3flsh.exe -o –sasadd 50030480070a8b00

4) Reboot the system

Flashing IT mode in UEFI shell: For all X9  X10 MB

1) Please copy all files from UEFI folder to bootable USB disk

2) Run SMC3216T.nsh and following the instruction to enter the last 9 digits/character of the SAS address.

3) If you are having problem to enter the SAS address, please use the command below to set the controller SAS address.

sas3flash.efi -o –sasadd “Controller SAS address”

For example: sas3flash.efi -o –sasadd 50030480070a8b00

4) Reboot the system